3.7-4.2GHz 6W VTX Video Transmitter Shrimp B

Key Features

- None standard frequency from 3.7G to 4 .2G for less congestion .

- Designed for long-range flights , up to 6W transmitting power, and provide multi-level adjustment.

- Efficient PA and power calibration to ensure power consistency across all channels .

- Customized digital PLL circuit , chip level integration , no frequency drift.

- Aluminum alloy three-layer shell structure and large

centrifugal fan provide sufficient heat dissipation to ensure stable output of transmitting power.

- 32 channels supports band switching within the 3.7–4 .2GHz range .

General Characteristics

| Frequency band | 3700-4320 MHz | Output power | 25mW/200mW/1W/6W |
| --- | --- | --- | --- |
| Channel | 32 CH | Frequency precision | ±200KHz (Typ.) |
| Modulation type | FM | Power Supply | DC 7-36V |
| Frequency control | PLL | Video input level | 1V±0.2Vp-p (typ.) |
| Control Protocol | IRC Tramp | S/N(For ±3MHz) | >55dBc |
| Antenna | SMA 50 Ohms | Operating Temperature | -10℃~+70℃ |
| Dimension | 44*28.6* 14.7mm | Weight | 25.3g |

Frequency table

| BAND | CH1 | CH2 | CH3 | CH4 | CH5 | CH6 | CH7 | CH8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BANDA | 3700 | 3720 | 3740 | 3760 | 3780 | 3800 | 3820 | 3840 |
| BAND b | 3860 | 3880 | 3900 | 3920 | 3940 | 3960 | 3980 | 4000 |
| BAND C | 4020 | 4040 | 4060 | 4080 | 4100 | 4120 | 4140 | 4160 |
| BAND d | 4180 | 4200 | 4220 | 4240 | 4260 | 4280 | 4300 | 4320 |

RF Power

| Value | 25 | 200 | 1000 | 5000 |
| --- | --- | --- | --- | --- |
| Power level | 25mW | 200mW | 1W | 5W |
| Gain | 14dBm | 23dBm±0.5 | 30dBm±0.5 | 38dBm±0.5 |

Button Operation

- Controlled using a single button , the button integrates RGB lighting , blue for band , red for channel , and green for power level. - When there is no operation , the RGB LED blinks cyclically, the number of times the RGB blinks represents the current VTX status (when the green LED is always on , the VTX enters PITMODE) . - Long press the button to enter VTX setting , at this time the RGB lights are lit alternately, release the button under the corresponding color light to enter the corresponding function setting. At this time , short press the button to adjust the value . For setup via the flight controller, connect the DATA pin and use the IRC Tramp protocol.

Attention

Antenna must be connected before use! Otherwise there is a risk of damage to the product. The product needs to be mounted reliably, avoiding vibration and high temperatures . The product has a built-in power filter, but still need to avoid excessive power supply interference .